How to estimate the wave length

Parameters:

- h= depth measure along the direction of wave propagation (at least three points);

- T= wave period; (for hypothesis constant along the propagation)

- L0= wave length on deep water;

- L= wave length according the value of depth;

- K= wave number.

:

K=2πL

L0=g∗T22π

L=g∗T22π∗Th2π∗hL

“a “ is for example 3% or other value

h; T

L0

First step

L=L0=Lc

(Li-Lc )/Lc % ≤ a

Yes

NO

L=Li

in next step: Lc=Li

Li = [gT2/(2])]*[Th(2*h/Lc)]

Example of calculation steps along the path choosen

| L0 | L1 | L2 | L3 | L4 | L5 | L6 | L7 | L8 | L9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [m] | [m] | [m] | [m] | [m] | [m] | [m] | [m] | [m] | [m] |
| 39.05 | 31.61 | 34.49 | 33.39 | 33.82 | 33.65 | 33.72 | 33.69 | 33.70 | 33.70 |
| d | d | d | d | d | d | d | d | d | d |
| [m] | [m] | [m] | [m] | [m] | [m] | [m] | [m] | [m] | [m] |
| … |  |  |  |  |  |  |  |  |  |

It is necessary that you verify that the condition is not as “shallow water

Then the ratio between depth d and Length L

Deep water d/L >0,5

Intermediate depth 0,05 <d/L<0,5

Shallow water d/L < 0,05